FPGA Acceleration of Transformer Attention

Presentation Notes - Fused Step 5 Hardware-Verified State

Target: Xilinx Alveo U55C | Model source: TinyLlama one-head attention vectors | Updated: May 4, 2026

# Executive Summary

Purpose: Give the audience the project in one minute.

We built and verified an isolated FPGA attention pipeline for one TinyLlama attention head.

The hardware path computes Q/K score, causal mask + scale, full-row softmax, and softmax @ V to produce attn_out.

Track A Step 5 is now hardware-verified: score + mask/scale are fused into score_mask_scale_u55c_kernel.

Synthetic sequence lengths S = 8, 64, 128, 256, 512 pass on the real U55C with the fused xclbin.

Real TinyLlama vector runs pass for the legacy single-tile case, S = 16, and S = 64.

All timing/utilization report artifacts and captured images are stored under docs/.

Speaker note: Emphasize that this is a correct isolated one-head attention accelerator, not full TinyLlama inference.

# Project Scope

Purpose: Clarify what was implemented and what was intentionally excluded.

Implemented offload boundary:

Q_rot_int8, K_rot_int8 -> score_scaled_fp32 -> score_softmax_fp32 -> attn_out_fp32

The fused branch computes raw INT32 scores internally but no longer writes raw scores back to HBM.

Precision choices:

Q and K are INT8 for bandwidth and compute density.

Score accumulation is INT32 inside the score path.

Mask/scale, softmax, V, and attn_out use FP32.

Intentionally out of scope:

Q/K/V projection GEMMs on FPGA

Live RoPE generation on FPGA

Full decoder-layer integration

Full TinyLlama tokens/sec inference benchmark

# Motivation

Purpose: Explain why attention was a useful FPGA target.

Transformer attention has regular dense math and heavy memory movement.

The score matrix grows as S x S, so sequence length quickly increases compute and storage demand.

FPGAs let us specialize data layout, arithmetic precision, tiling, HBM placement, and kernel orchestration.

This project tests the design on a real U55C, including hardware emulation, real-card execution, and post-route reports.

# Current Architecture

Purpose: Show how data moves through the fused verified pipeline.

Host/XRT prepares synthetic or exported real TinyLlama Q/K/V vectors.

Tile shape: Q tile = 8 x 64 INT8, K tile = 64 x 64 INT8, score tile = 8 x 64.

Full sequence coverage uses ceil(S/8) x ceil(S/64) score tiles per head.

Fused pre-softmax path: score_mask_scale_u55c_kernel writes scaled FP32 logits directly.

Full-row softmax is required for S > 64 because each query row must normalize across all keys.

The fused xclbin uses HBM banks [0], [1], and [3]-[7]; HBM[2] is no longer needed for raw-score staging.

Figure: one-head attention pipeline diagram.

Figure: refreshed Vitis device/HBM map capture for the fused build.

# Track Status

Use this slide to show the current completed scope and the remaining future work.

| Track | Status | Notes |
| --- | --- | --- |
| Track A: speedup + tiling | Steps 1-5 hardware-verified | Includes packed I/O, merged mask+scale, tiling, full-row softmax, pass-1 double buffering, and fused score+mask/scale. |
| Track B: softmax @ V | Complete | V weighted-sum kernel integrated; synthetic S=8..512 and vector modes verify final attn_out. |
| Track C: real TinyLlama inputs | Complete for current scope | TinyLlama Q/K/V extraction, INT8 Q/K quantization, real-vector export, and U55C runs for single-tile, S=16, S=64. |
| Track D: baselines/results | Staged design complete; fused retiming partially captured | Fused hardware timings are available from AGENTS/U55C run logs; Track D tables should be refreshed for fused-vs-staged comparison. |
| Future integration | Not implemented | Full TinyLlama subgraph, KV-cache-aware decode, and decoder-layer integration remain future work. |

# Implementation Highlights

Purpose: Summarize the important engineering decisions.

64-bit packed Q/K/score I/O was the first major score-kernel latency improvement.

Full-row softmax made S > 64 correct by normalizing across the entire row.

The V weighted-sum kernel completes one-head attention output instead of stopping at probabilities.

Track A Step 5 fused score + mask/scale, eliminating the raw-score HBM write/read round trip.

The fused kernel passed HLS csim/csynth, hw_emu, and real U55C runs.

The fused hardware xclbin UUID is 56cd611d-8c32-c19b-f5ef-358bed40d459.

# Verification Strategy

Purpose: Demonstrate that correctness was checked at multiple layers.

Python reference verifies full-sequence tiling at S = 8, 64, 128, 256, 512.

Local C++ benches verify individual HLS kernels against exported vectors.

Fused score_mask_scale_u55c_kernel: HLS csim PASS and csynth PASS.

Fused hw_emu passed vector mode and S=8 sequence mode.

Fused real U55C synthetic sweep passed S = 8, 64, 128, 256, 512.

Fused real U55C TinyLlama vector runs passed for real_tinyllama_tile, real_tinyllama_s16, and real_tinyllama_s64.

Tolerance policy:

Scaled logits and softmax: 1e-4

Synthetic/quantized attn_out: 1e-4

Real TinyLlama PyTorch float reference attn_out: 1e-3

# Correctness Results

Purpose: Give the pass/fail headline without drowning the slide in logs.

Fused real U55C --vectors sim/attention_score_tile passed with final attn_out verification.

Fused real U55C synthetic S=8, 64, 128, 256, 512 all printed Tiled sequence verification PASSED and Attention output verification PASSED.

Fused real U55C TinyLlama vector runs passed for sim/real_tinyllama_tile, sim/real_tinyllama_s16, and sim/real_tinyllama_s64.

Post-route timing met all user timing constraints.

# Fused Hardware Timing Results

These are real U55C fused-branch timings for the isolated one-head attention block, not full-model tokens/sec.

| Case | Fused FPGA total ms | Previous staged total ms | Notes |
| --- | --- | --- | --- |
| Synthetic S=8 | 0.296 | 0.803 | Fused path removes one pre-softmax kernel launch and raw-score HBM staging. |
| Synthetic S=64 | 2.364 | 2.864 | Same final attn_out verification. |
| Synthetic S=128 | 5.484 | 6.992 | First multi-K-chunk comparison point. |
| Synthetic S=256 | 17.307 | 20.570 | Fused path improves total time but softmax/V stages remain staged. |
| Synthetic S=512 | 60.328 | 72.941 | Largest tested synthetic length. |
| real_tinyllama_tile | 0.635 | 0.258 | Small single-tile runs are sensitive to fixed overhead/cold effects. |
| real_tinyllama_s16 | 0.569 | 1.488 | Real TinyLlama full-sequence vector run. |
| real_tinyllama_s64 | 2.004 | 3.094 | Real TinyLlama full-sequence vector run. |

Interpretation: Step 5 improves the larger synthetic and full-sequence real-vector cases by reducing staged pre-softmax overhead.

The design is still staged after pre-softmax: full-row softmax and softmax @ V continue to use separate kernels and HBM buffers.

# CPU/GPU/FPGA Baseline Context

The CPU/GPU baseline table is from the staged Track D results and is still useful context. The fused FPGA column should be used for current hardware timing.

| S | CPU full attn ms | GPU full attn ms | Staged FPGA total ms | Fused FPGA total ms |
| --- | --- | --- | --- | --- |
| 8 | 0.0322 | 0.3349 | 0.803 | 0.296 |
| 64 | 0.2920 | 0.2507 | 2.864 | 2.364 |
| 128 | 2.5009 | 0.2998 | 6.992 | 5.484 |
| 256 | 3.9523 | 0.2726 | 20.570 | 17.307 |
| 512 | 12.3290 | 0.2461 | 72.941 | 60.328 |

The GPU baseline was collected on a separate Windows RTX 3050 Laptop GPU and should not be presented as same-host U55C data.

Even after fusion, this isolated one-head workload is still slower than CPU/GPU baselines; the value of Step 5 is architectural progress toward fewer launches and less HBM staging.

# Fused Post-Route Utilization

Use the refreshed post-route reports under docs/ for final fused FPGA utilization numbers.

| Scope | LUT | REG | BRAM | URAM | DSP |
| --- | --- | --- | --- | --- | --- |
| User kernels | 60,983 | 70,401 | 110 | 2 | 405 |
| Full routed design | 191,648 CLB LUTs | 253,388 CLB registers | 309.5 Block RAM tiles | 2 | 409 |

Full routed design usage: 14.70% CLB LUTs, 9.72% registers, 15.35% BRAM, 0.21% URAM, 4.53% DSP.

The V weighted-sum kernel still dominates DSP usage: 320 DSP. The fused score_mask_scale kernel uses 67 DSP and 23 BRAM.

Post-route timing closed: WNS 0.003 ns, TNS 0, WHS 0.009 ns, no failing setup/hold endpoints.

Figure: refreshed fused post-route utilization capture.

Figure: refreshed fused post-route timing summary capture.

# Report Artifacts Under docs/

All timing, utilization, xclbin, system-estimate, and captured-image artifacts used for the presentation are stored under docs/.

| File | Use in presentation |
| --- | --- |
| docs/track_d_results.md | Staged CPU/GPU/FPGA timing tables and interpretation; refresh this later with fused comparison tables. |
| docs/PostRouteKernelUtilization.rpt | Per-kernel fused routed utilization. Use this to explain DSP/BRAM by kernel. |
| docs/PostRouteFullUtilization.rpt | Full fused routed U55C utilization including platform resources. Use this for headline percentages. |
| docs/PostRouteSLRUtilization.rpt | SLR resource distribution and SLL usage. Useful for placement/spread discussion. |
| docs/PostRouteTimingSummary.rpt | Fused post-route timing closure. Source for WNS/TNS/WHS and no failing endpoints. |
| docs/PostRouteTimingSummary.rpv / .rpx | Vivado report viewer/project artifacts for deeper timing inspection. |
| docs/PostRouteUtilization.xlsx | Spreadsheet copy of utilization for charts or quick slide edits. |
| docs/system_estimate_attention_score_chain.xtxt | Full fused xclbin system-estimate output. |
| docs/system_estimate_score_mask_scale_u55c_kernel.xtxt | Fused pre-softmax kernel system estimate. |
| docs/system_estimate_softmax_u55c_kernel.xtxt | Legacy tile softmax system estimate. |
| docs/system_estimate_softmax_full_row_u55c_kernel.xtxt | Full-row softmax system estimate. |
| docs/system_estimate_v_weighted_sum_u55c_kernel.xtxt | V weighted-sum system estimate. |
| docs/attention_score_chain_xclbin_info.txt | xclbin content, UUID, kernels, clocks, and memory topology. |
| docs/attention_score_chain_xclbin_link_summary.txt | Vitis link summary and command/config mirrors. |
| docs/system_diagram_fused.json | Fused system-diagram JSON preserved from Vitis save-temps. |
| docs/vpp_*_guidance.html | Vitis compile/link guidance pages for each kernel and link step. |
| docs/PlatformDiagram.pdf and docs/SystemDiagram.pdf | Generated Vitis platform/system diagrams. |
| docs/attention_architecture.png | Presentation architecture figure. |
| docs/DeviceMap.png | Device/HBM map capture. |
| docs/post_route_utilization.png | Utilization screenshot for slides. |
| docs/timing_summary.png | Timing closure screenshot for slides. |

# Challenges

Purpose: Explain what made the project nontrivial.

Softmax is row-global: S > 64 cannot be normalized correctly one 8 x 64 tile at a time.

The host must assemble full S-wide rows before launching full-row softmax.

Staged kernels are easy to debug but create extra HBM traffic and launch overhead.

Fusing score + mask/scale required preserving correctness while changing the host and xclbin kernel graph.

Real TinyLlama data required careful Q/K/V extraction, RoPE alignment, grouped-query head mapping, and quantization validation.

# Current Limitations

Purpose: Keep the claims honest.

This is one selected attention head, not all TinyLlama heads and layers.

Q/K/V projection and RoPE are still produced by software/export scripts, not FPGA kernels.

The fused design improves over the staged FPGA path but is still slower than CPU/GPU for this isolated one-head workload.

Real TinyLlama full-sequence FPGA tests are currently S = 16 and S = 64. Synthetic --seq-len tests have already covered S = 128, 256, and 512; only larger real TinyLlama vector directories remain optional follow-up.

Full TinyLlama integration and model-level tokens/sec are future work.

# Next Steps

Purpose: Show the immediate engineering path after the current state.

Use docs/track_d_results.md as the current fused-vs-staged timing and utilization source.

Optional: generate and run larger real TinyLlama vector directories such as real_tinyllama_s128, real_tinyllama_s256, and real_tinyllama_s512 if the final demo needs broader real-input coverage.

Investigate additional fusion after softmax/V only if the algorithmic data dependencies can be handled cleanly.

Connect this block into a fuller TinyLlama attention subgraph.

Longer-term: add KV-cache-aware decode and eventually integrate into a decoder-layer flow.

# Suggested Slide Order

Title and project goal

Why attention is a bottleneck

Offload boundary and what is out of scope

Architecture diagram and device map

Track A-D implementation status

Verification flow

Correctness results

Fused hardware timing results

CPU/GPU/FPGA baseline context

Fused post-route utilization and timing closure

Report artifacts/source files under docs/

Limitations and honest interpretation

Next steps

# Appendix: Key Files To Reference

AGENTS.md - latest project-state handoff and fused Step 5 verification summary

docs/implementation_checklist.md - track status and implementation plan

docs/track_d_results.md - fused-vs-staged CPU/GPU/FPGA timing and interpretation

docs/PostRouteKernelUtilization.rpt - fused per-kernel routed utilization

docs/PostRouteFullUtilization.rpt - fused full routed design utilization

docs/PostRouteTimingSummary.rpt - fused timing closure details

docs/attention_score_chain_xclbin_info.txt - fused xclbin UUID, kernels, clocks, and topology

docs/attention_architecture.drawio / .png - architecture diagram

host/attention_score_chain_xrt.cpp - XRT host orchestration

hls/score_and_mask_scale/ - Track A Step 5 fused kernel